|  | ## Hybrid Stepping Motor ## Model: 35BYG |
| --- | --- |

| General Specifications |  | Model: 35BYG |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resistance | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 35BYG101 | 1.8 | 20 | 0.05 | 6 | 0.4 | 25 | 15 | 8 | 60 | 100 |
| 35BYG102 | 1.8 | 20 | 0.04 | 6 | 0.5 | 13 | 3.5 | 8 | 60 | 100 |
| 35BYG103 | 1.8 | 20 | 0.065 | 4 | 0.73 | 9 | 13 | 8 | 60 | 100 |
| 35BYG202A | 1.8 | 26 | 0.03 | 6 | 0.16 | 18 | 5.2 | 10 | 60 | 0.13 |
| 35BYG201 | 1.8 | 26 | 0.035 | 6 | 0.25 | 45 | 13 | 10 | 60 | 130 |
| 35BYG203 | 1.8 | 26 | 0.09 | 4 | 1.5 | 1.6 | 1.4 | 10 | 60 | 130 |
| 35BYG301 | 1.8 | 28 | 0.08 | 4 | 0.5 | 20 | 19 | 10 | 80 | 140 |
| 35BYG302 | 1.8 | 28 | 0.09 | 6 | 0.4 | 28 | 10 | 10 | 80 | 140 |
| 35BYG303 | 1.8 | 28 | 0.05 | 4 | 0.54 | 15 | 7.5 | 10 | 80 | 140 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 39BYG |
| --- | --- |

| General Specifications |  | Model: 39BYG |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resistance | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 39BYG001 | 1.8 | 20 | 0.05 | 6 | 0.4 | 25 | 40 | 11 | 50 | 0.12 |
| 39BYG002 | 1.8 | 20 | 0.065 | 6 | 0.5 | 13 | 12 | 11 | 50 | 0.12 |
| 39BYG003 | 1.8 | 20 | 0.08 | 4 | 0.48 | 25 | 23 | 11 | 50 | 0.12 |
| 39BYG300 | 1.8 | 20 | 0.1 | 4 | 0.48 | 25 | 26 | 11 | 50 | 0.12 |
| 39BYG201 | 1.8 | 34 | 0.11 | 6 | 0.16 | 75 | 50 | 20 | 120 | 0.18 |
| 39BYG204 | 1.8 | 34 | 0.22 | 4 | 0.4 | 30 | 35 | 20 | 120 | 0.18 |
| 39BYG205 | 1.8 | 34 | 0.21 | 4 | 0.4 | 24 | 25 | 20 | 120 | 0.18 |
| 39BYG210 | 1.8 | 34 | 0.16 | 6 | 0.8 | 7.6 | 4.5 | 20 | 120 | 0.18 |
| 39BYG400 | 1.8 | 38 | 0.2 | 6 | 0.8 | 7.2 | 3.5 | 24 | 180 | 0.2 |
| 39BYG601 | 1.8 | 44 | 0.28 | 4 | 0.30 | 40 | 100 | 40 | 250 | 0.25 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 39BYGHM |
| --- | --- |

| General Specifications: |  | Model: 39BYGHM |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resistance | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 39BYGHM001 | 0.9 | 20 | 0.028 | 6 | 0.50 | 24.0 | 35.0 | 8 | 21 | 0.12 |
| 39BYGHM203 | 0.9 | 24 | 0.100 | 4 | 0.67 | 6.0 | 10.2 | 11 | 28 | 0.15 |
| 39BYGHM201 | 0.9 | 26 | 0.030 | 4 | 0.40 | 13.3 | 7.2 | 11 | 28 | 0.15 |
| 39BYGHM202 | 0.9 | 26 | 0.095 | 4 | 0.64 | 9.0 | 6.4 | 11 | 28 | 0.15 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 42BYGHM |
| --- | --- |

| General Specifications: |  | Model: 42BYGHM |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resist. | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 42BYGHM100W-01A | 0.9 | 28 | 0.1 | 4 | 0.425 | 8 | 11 | 25 | 150 | 0.16 |
| 42BYGHM203 | 0.9 | 34 | 0.158 | 6 | 0.3 | 40 | 37 | 34 | 200 | 0.22 |
| 42BYGHM206 | 0.9 | 34 | 0.14 | 6 | 1 | 3.6 | 3.3 | 35 | 200 | 0.22 |
| 42BYGHM207 | 0.9 | 34 | 0.23 | 4 | 2.4 | 1 | 1.4 | 35 | 200 | 0.22 |
| 42BYGHM601 | 0.9 | 40 | 0.26 | 6 | 1.2 | 3.3 | 3.2 | 54 | 250 | 0.28 |
| 42BYGHM604 | 0.9 | 40 | 0.36 | 4 | 1.68 | 1.65 | 3.2 | 54 | 250 | 0.28 |
| 42BYGHM608 | 0.9 | 40 | 0.26 | 6 | 2.13 | 1.05 | 6 | 54 | 220 | 0.28 |
| 42BYGHM802 | 0.9 | 48 | 0.32 | 6 | 0.8 | 7.5 | 6.3 | 68 | 280 | 0.35 |
| 42BYGHM805A | 0.9 | 48 | 0.43 | 4 | 1 | 3.5 | 7.6 | 68 | 250 | 0.35 |
| 42BYGHM807 | 0.9 | 48 | 0.32 | 6 | 1.2 | 3.6 | 4.2 | 68 | 250 | 0.35 |
| 42BYGHM809 | 0.9 | 48 | 0.42 | 4 | 1.68 | 1.65 | 3.5 | 68 | 250 | 0.35 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 42BYG |
| --- | --- |

| General Specifications: |  | Model: 42BYG |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resist. | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 42BYG802 | 1.8 | 48 | 0.29 | 4 | 0.9 | 4.2 | 6.5 | 36 | 250 | 0.3 |
| 42BYGS200 | 3.75 | 34 | 0.077 | 6 | 0.2 | 60 | 20 | 20 | 150 | 0.2 |
| 42BYGS201A | 3.75 | 34 | 0.065 | 4 | 0.45 | 19.2 | 9.6 | 20 | 150 | 0.2 |
| 42BYGS301B-01A | 3.75 | 40 | 0.1 | 4 | 1.2 | 3 | 4 | 25 | 170 | 0.23 |
| 42BYG201 | 1.8 | 34 | 0.09 | 4 | 0.6 | 7 | 6 | 20 | 150 | 0.2 |
| 42BYG202 | 1.8 | 34 | 0.13 | 6 | 0.45 | 20 | 12 | 20 | 150 | 0.2 |
| 42BYG203 | 1.8 | 34 | 0.1 | 2 6 | 0.3 | 40 | 18 | 20 | 150 | 0.2 |
| 42BYG601 | 1.8 | 38 | 0.2 | 6 | 0.4 | 30 | 23 | 22 | 150 | 0.2 |
| 42BYG701 | 1.8 | 44 | 0.28 | 4 | 0.3 | 40 | 80 | 30 | 220 | 0.26 |
| 42BYG801 | 1.8 | 48 | 0.24 | 4 | 0.4 | 11.2 | 24 | 36 | 250 | 0.3 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 42BYGHW |
| --- | --- |

| General Specifications |  | Model: 42BYGHW |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resist. | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 42BYGHW300 | 1.8 | 31 | 0.1 | 6 | 1.1 | 3.7 | 1.8 | 25 | 180 | 0.18 |
| 42BYGHW203A | 1.8 | 34 | 0.1 | 5 | 0.16 | 60 | 35 | 34 | 200 | 0.2 |
| 42BYGHW213-01 | 1.8 | 34 | 0.24 | 5 | 0.6 | 20 | 13 | 34 | 200 | 0.2 |
| 42BYGHW602-01 | 1.8 | 40 | 0.26 | 6 | 0.4 | 30 | 22 | 54 | 220 | 0.24 |
| 42BYGHW609 | 1.8 | 40 | 0.42 | 4 | 1.7 | 1.5 | 3.2 | 54 | 220 | 0.24 |
| 42BYGHW700B | 1.8 | 44 | 0.28 | 4 | 0.3 | 40 | 67 | 50 | 250 | 0.3 |
| 42BYGHW801 | 1.8 | 48 | 0.32 | 6 | 0.4 | 30 | 25 | 68 | 280 | 0.34 |
| 42BYGHW xxx 外止口电机 |  |  |  |  |  |  |  |  |  |  |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 42BYGH |
| --- | --- |

| General Specifications |  | Model: 42BYGH |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resist. | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 42BYGH201-01 | 1.8 | 34 | 0.095 | 6 | 0.16 | 75 | 55 | 34 | 200 | 0.2 |
| 42BYGH218 | 1.8 | 34 | 0.21 | 4 | 1.5 | 1.3 | 1.8 | 34 | 200 | 0.2 |
| 42BYGH607-01 | 1.8 | 40 | 0.45 | 4 | 0.8 | 10 | 20 | 54 | 220 | 0.24 |
| 42BYGH610-01 | 1.8 | 40 | 0.28 | 6 | 1.2 | 3 | 2.8 | 54 | 220 | 0.24 |
| 42BYGH701 | 1.8 | 44 | 0.3 | 4 | 0.1 | 213 | 360 | 50 | 250 | 0.3 |
| 42BYGH707B | 1.8 | 44 | 0.35 | 4 | 1 | 4.4 | 9 | 50 | 250 | 0.3 |
| 42BYGH805-01 | 1.8 | 48 | 0.42 | 6 | 0.8 | 13.5 | 11.3 | 68 | 280 | 0.34 |
| 42BYGH810 | 1.8 | 48 | 0.45 | 4 | 1.2 | 3.2 | 5.6 | 68 | 280 | 0.34 |
| 42BYGH xxx 内止口电机 |  |  |  |  |  |  |  |  |  |  |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 60BYGH |
| --- | --- |

| General Specifications |  | Model: 60BYGH |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resistance | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 60BYGH801A | 1.8 | 87 | 1.25 | 6 | 2 | 2.8 | 6 | 570 | 0.4 | 1.34 |
| 60BYGH802 | 1.8 | 87 | 1.95 | 4 | 2.5 | 2 | 9.5 | 570 | 0.4 | 1.34 |
| 60BYGH803 | 1.8 | 87 | 1.13 | 6 | 1 | 10 | 19.5 | 570 | 0.4 | 1.34 |
| 60BYGH804 | 1.8 | 87 | 0.86 | 8 | 4 | 0.35 | 1 | 570 | 0.4 | 1.34 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 57BYGHM |
| --- | --- |

| General Specifications |  | Model: 57BYGHM |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resistance | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 57BYGHM011 | 0.9 | 41 | 0.32 | 6 | 1 | 5.7 | 9 | 120 | 0.21 | 0.45 |
| 57BYGHM007 | 0.9 | 41 | 0.22 | 6 | 2 | 1.55 | 2.2 | 120 | 0.21 | 0.45 |
| 57BYGHM004-01 | 0.9 | 41 | 0.3 | 4 | 2.8 | 0.7 | 1.9 | 120 | 0.21 | 0.45 |
| 57BYGHM200 | 0.9 | 51 | 0.7 | 4 | 1 | 2.9 | 12 | 240 | 0.26 | 0.56 |
| 57BYGHM401 | 0.9 | 54 | 0.9 | 6 | 1 | 7.4 | 10 | 260 | 380 | 0.7 |
| 57BYGHM402-2 | 0.9 | 56 | 0.9 | 6 | 2 | 1.8 | 3.8 | 300 | 0.4 | 0.7 |
| 57BYGHM412 | 0.9 | 56 | 1 | 4 | 2.8 | 0.98 | 4 | 300 | 0.4 | 0.7 |
| 57BYGHM601 | 0.9 | 76 | 1.3 | 6 | 1 | 8.6 | 14 | 460 | 680 | 1 |
| 57BYGHM605 | 0.9 | 78 | 1.35 | 6 | 3 | 1 | 1.6 | 480 | 0.68 | 1 |
| 57BYGHM606 | 0.9 | 78 | 1.45 | 4 | 2 | 2.2 | 11.6 | 480 | 0.68 | 1 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 57BYG |
| --- | --- |

| General Specifications |  | Model: 57BYG |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resistance | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 57BYG001 | 1.8 | 41 | 0.39 | 6 | 0.38 | 32 | 30 | 57 | 0.18 | 0.35 |
| 57BYG003 | 1.8 | 41 | 0.3 | 6 | 0.6 | 20 | 18 | 57 | 0.18 | 0.35 |
| 57BYG006 | 1.8 | 41 | 0.3 | 6 | 1.1 | 3.6 | 4.1 | 57 | 0.18 | 0.35 |
| 57BYG201 | 1.8 | 51 | 0.5 | 6 | 0.3 | 80 | 97 | 110 | 0.35 | 0.5 |
| 57BYG214 | 1.8 | 51 | 0.49 | 6 | 1.3 | 3.1 | 4.4 | 110 | 0.35 | 0.5 |
| 57BYG223 | 1.8 | 51 | 0.62 | 4 | 2 | 1.3 | 3.8 | 110 | 0.35 | 0.5 |
| 57BYG402 | 1.8 | 57 | 0.6 | 6 | 0.6 | 20 | 30 | 145 | 500 | 0.6 |
| 57BYG405 | 1.8 | 57 | 0.6 | 6 | 1.2 | 5 | 8 | 135 | 0.42 | 0.65 |
| 57BYG410 | 1.8 | 57 | 0.58 | 6 | 2.4 | 1 | 1.7 | 135 | 0.42 | 0.65 |
| 57BYG601 | 1.8 | 76 | 0.6 | 6 | 0.6 | 20 | 22 | 200 | 0.72 | 0.9 |
| 57BYG606 | 1.8 | 76 | 0.9 | 6 | 1.5 | 3.6 | 6.2 | 200 | 0.72 | 0.95 |
| 57BYG610-01 | 1.8 | 76 | 0.86 | 6 | 2 | 1.68 | 3.5 | 200 | 0.72 | 0.95 |
| 57BYG100-001 | 1.8 | 100 | 1.2 | 4 | 2 | 1.05 | 4.5 | 300 | 0.95 | 1.25 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 57BYGH |
| --- | --- |

| General Specifications |  | Model: 57BYGH |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resistance | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 57BYGH001A | 1.8 | 41 | 0.35 | 4 | 0.5 | 12 | 23 | 120 | 0.21 | 0.45 |
| 57BYGH009A | 1.8 | 41 | 0.39 | 6 | 1 | 5.7 | 6.5 | 120 | 0.21 | 0.45 |
| 57BYGH016A | 1.8 | 41 | 0.35 | 6 | 2 | 1.4 | 1.6 | 120 | 0.21 | 0.45 |
| 57BYGH209 | 1.8 | 51 | 0.6 | 6 | 0.38 | 32 | 38 | 275 | 0.36 | 0.65 |
| 57BYGH206 | 1.8 | 51 | 0.72 | 6 | 1 | 6.6 | 8.2 | 275 | 0.36 | 0.65 |
| 57BYGH224 | 1.8 | 51 | 0.72 | 6 | 3 | 0.74 | 0.9 | 275 | 0.36 | 0.65 |
| 57BYGH401A | 1.8 | 56 | 0.9 | 4 | 0.6 | 10 | 32 | 300 | 0.4 | 0.7 |
| 57BYGH406A | 1.8 | 56 | 0.9 | 4 | 1 | 6.2 | 11 | 300 | 0.4 | 0.7 |
| 57BYGH436 | 1.8 | 56 | 0.9 | 6 | 3 | 0.8 | 1.2 | 300 | 0.4 | 0.7 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 57BYGH |
| --- | --- |

| General Specifications |  | Model: 57BYGH |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Phase Resist. | Phase Inductance | Rotor Inertia | Detent Torque | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (Ω) | (mH) | (g.cm2) | (g.cm) | (Kg) |
| 57BYGH502 | 1.8 | 67 | 1 | 6 | 2 | 1.7 | 2.5 | 360 | 0.50 | 0.82 |
| 57BYGH603 | 1.8 | 78 | 1.6 | 6 | 1 | 8.6 | 18 | 480 | 0.68 | 1.00 |
| 57BYGH613 | 1.8 | 78 | 1.35 | 6 | 2 | 2.25 | 4.1 | 480 | 0.68 | 1.00 |
| 57BYGH646 | 1.8 | 78 | 1.4 | 6 | 4 | 0.7 | 1.5 | 480 | 0.68 | 1.00 |
| 57BYGH803 | 1.8 | 84 | 1.8 | 6 | 4 | 0.7 | 1.6 | 530 | 0.65 | 1.13 |
| 57BYGH804 | 1.8 | 84 | 1.8 | 4 | 6.6 | 0.2 | 1.2 | 530 | 0.65 | 1.13 |
| 57BYGH115-001 | 1.8 | 115 | 2.6 | 4 | 1 | 15.6 | 76 | 810 | 0.89 | 1.55 |
| 57BYGH115-007 | 1.8 | 115 | 2 | 6 | 4.2 | 1 | 2.1 | 810 | 0.89 | 1.55 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |

|  | ## Hybrid Stepping Motor ## Model: 85BYGH |
| --- | --- |

| General Specifications |  | Model: 85BYGH |
| --- | --- | --- |
| Step Accuracy | ±5% |  |
| Temperature Rise | 80℃ Max. |  |
| Ambient Temperature Range | 20℃~+50℃ |  |
| Insulation Resistance | 100MΩ Min.50V DC |  |
| Dielectric Strength | 500V AC 1minute |  |
| Radial Play | 0.02mm Max.(450g Load) |  |
| End Play | 0.08mm Max.(450g Load) |  |
| Rotation | CW (See from Front Flange) |  |

| ##### Main Technical Data: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Step Angle | Motor Length | Holding Torque | Lead Wire | Rated Current | Rotor Inertia | Motor Weight |
|  | (°) | (mm) | (N.m) | (NO.) | (A) | (g.cm2) | (Kg) |
| #### 85BYGH450A-001 | 1.8 | 75 | 2.1 | 8 | 2 | 1.4 | 1.7 |
| 85BYGH450A-002 | 1.8 | 75 | 2.1 | 8 | 3 | 1.4 | 1.7 |
| 85BYGH450A-003 | 1.8 | 75 | 2.1 | 4 | 3.2 | 1.4 | 1.7 |
| 85BYGH450B-002 | 1.8 | 113 | 4.1 | 8 | 2 | 2.7 | 2.8 |
| 85BYGH450B-003 | 1.8 | 113 | 4.1 | 8 | 3 | 2.7 | 2.8 |
| 85BYGH450B-005 | 1.8 | 113 | 4.1 | 8 | 4.5 | 2.7 | 2.8 |
| 85BYGH450C-001B | 1.8 | 151 | 6.4 | 8 | 2 | 3.6 | 5 |
| 85BYGH450C-002 | 1.8 | 151 | 6.4 | 8 | 4 | 3.6 | 5 |
| 85BYGH450C-003 | 1.8 | 151 | 6.4 | 8 | 4.2 | 3.6 | 5 |
| 85BYGH450D-001 | 1.8 | 94 | 7 | 4 | 3 | 1.7 | 2.1 |
| 85BYGH450D-002 | 1.8 | 94 | 4.3 | 8 | 4 | 1.7 | 2.1 |

| ##### Outline Drawing: | ##### Connecting Diagram: |
| --- | --- |